Below is a comparison between DART-MX6 and Raspberry Pi Zero 2 W.

| DART-MX6 | Raspberry Pi Zero 2 W |
| --- | --- |
| Freescale i.MX6 series SoC (Dual /Quad ARM® Cortex™-A9 Core, 800Mhz) | 1GHz quad-core 64-bit Arm Cortex-A53 CPU |
| TI WiLink8 2.4/5GHz WLAN (802.11 a/b/g/n) / BT-BLE 5.1 with CSA2 support and optional MIMO | 2.4GHz 802.11 b/g/n wireless LAN & Bluetooth 4.2 (BLE) |
| 24-bit Parallel LCD interface 2 x LVDS display interface HDMI V1.4 interface 1 x MIPI DSI | Mini HDMI® port and micro–USB On-The-Go (OTG) port |
| Up to 128GB eMMC storage | microSD card slot (Depends on user) |
| Up to 2GB LPDDR2 RAM | 512MB SDRAM |
| Serial interfaces ( SPI , I2C, UART, I2S, SPDIF, ESAI) | HAT-compatible 40-pin header footprint (unpopulated) |
| 50mm x 20mm, 2x80 + 1x50 pin Board to Board Connectors | 65mm x 30mm |
| $60-70 | $16-80 |

Sources:(Canakit; Variscite, 2023; Raspberry Ltd)

Rationale

I’m comparing between 2 microprocessors that are best suited for the situation. They are similar in size and pricing. I believe DART-MX6 is the better choice, this module is designed for low power applications. Which is perfect for our configuration with 2 LED, LCD, while using I2C and UART interfaces. The DART-MX6 has lots of display ports for us to select from and it even has a dedicated interface for LCDs. It also has greater flexible connectivity with WI-FI with 2.4/5GHz bandwidths accepted. DART-MX6 has more RAM compared to the Raspberry Pi for running our program. The transfer speeds solder is fast and reliable they also consume less power. Although it falls a bit short on raw processing power. If using the module with the quad core It should be able to handle multithreading just as well as the Pi. The Dart-MX6 also has a smaller form factor which is a benefit for our thermostat application. Raspberry Pi Zero 2 W is a great choice as well but more overpower than what is necessary to transfer data to the cloud and run concurrent tasks.

References

CanaKit. (n.d.). Raspberry Pi Zero 2 W. CanaKit. https://www.canakit.com/raspberry-pi-zero-2-w.html?cid=usd&src=raspberrypi

Variscite. (2023, February 27). VARISCITE LTD.DART-MX6 v1.2/v1.3x DatasheetFreescale i.MX6TM - based System-on-Module. Variscite.com. https://variscite.com/wp-content/uploads/2017/11/DART-MX6-Datasheet.pd

Raspberry Ltd. (n.d.). Raspberry Pi Zero 2 W. Raspberry Pi. https://www.raspberrypi.com/products/raspberry-pi-zero-2-w/